Contents

Construct electrode components	1

Common REF block – 3 channels	1

Common REF block – 4 channels	2

For Single Wire Implants:	2

Cortical Screws	3

EMG Leads	3

For Bipolar Twisted wires	3

Perform stereotactic EEG Implant Surgery	4

Prep	4

Surgery	6

References	11

# Construct electrode components

## Common REF block – 3 channels

- Cut 4x2 block. File down edge of 4th row to make smooth. Remove silver pins from block.

- For comm-REF pins, twist bare end of two 2.0 cm silver wires together.

- Paint twist and pin tails with flux.

- Solder twisted end of silver wire to end of right angle pin #4.

- Solder other end of first 2.0 cm silver wire to right angle pin #6.

- Solder other end of second 2.0 cm silver wire to 0.10” screw.

- Verify continuity.

- On microconnector, paint GND edge with whiteout (pin #1).

- Make electrodes for surgery: solder ends of 2.0 cm silver wire to right angle pin and 0.10” crew. Make 4x.

## Common REF block – 4 channels

- Make 5x2 block. Remove silver pins from block.

- Cut 2.0 cm silver wire.

- Apply flux to wrapped screw. Allow to dry 10 min.

- Solder one end to 0.10” screw

- Solder other end to right angled gold tail

- Insert into #5 slot

- Insert two tails into #7/8 slots.

- For 5-7-8 common reference block: twist 2 silver wires and solder contact to one right angle tail. Solder end of one silver wire to 7/8 tails and the other silver wire to a 0.10” cortical screw.

## For Single Wire Implants:

- Cut 7 cm stainless steel wire

- Fold in half. Insert into block to find length of coating to remove. Fold to make crease.

- Burn off ~0.5cm from bend. This should be the depth of the micropin block.

- Sonicate to remove coating (~5min). Verify under dissection scope.

- Check Continuity

- Insert in block. Push tail-less pins into block to secure single wire.

- Use wirecutter to clip and separate wires.

- Verify circuit with “Continuity tester” mode of multimeter.

- For common reference PreAmp  insert tail-less pins into #7/#8 slot

## Cortical Screws

- Clamp right angle pins.

- Coat tail with flux. Allow to dry 10 min.

- Strip ~1mm Teflon off from silver wire.

- Solder silver wire to tail of gold pin.

- Cut 2.0 cm length of silver wire.

- Strip ~2mm Teflon off from other end of silver wire.

- Curl around cortical screw head.

- Add flux to silver curl. Allow to dry 10 min.

- Solder.

- Verify with “continuity tester”.

## EMG Leads

- Clamp right angle pins (2x).

- Coat tails with flux.

- Strip ~1mm Teflon off from silver wire.

- Solder 2.0 cm silver wire to tail of pin (2x).

- Insert pins into slot pin #3 & #6 (3chan) or #4 & #8 (4chan).

- Point to posterior direction (away from row B) and twist wires together but leave ends pointing away (don’t want to short).

- Verify with “continuity tester”.

## For Bipolar Twisted wires

- Specifications

  - 0.0035” = 88.9 microns in diameter

  - Inspect untwisted ends to ensure void of kinks.

- Fasten wires to block

  - Heat untwisted ends to strip coating (lacquer). Use flat forceps to strip burnt coating.

  - Cut 2.5 cm from start of twist

- Make Z bend in twisted wire.

  - Using slim flat forceps, grip at slightly above 2mm. Twist and bend to make Z.

  - Want Z bend to be in one plane and near 90° bend

- Verify circuit with “Continuity tester” mode of multimeter.

# Perform stereotactic EEG Implant Surgery

## Prep

- Turn on heat sterilizer.

- Sonicate EEG implant (2 twisted wires, #1, 2, 5, 6 bone screws)

Left side (bead sterilizer, eye ointment, beta-iodine)

Left side: bead sterilizer, eye ointment, beta-iodine

Surgical Tools: bent serrated forceps and microscissors; drill bit, hemostat & plastic cup (mix grip cement); flathead screwdriver & straight forceps; cement applicator and short transfer pipette, sterile cotton swabs (x2), sterile gauze (x2), grip cement and solvent.

Stereotactic frame setup: mouse in frame on thermal pad. Surgical gloves, right arm w/ drill, left arm with clamp (to hold implant), multimeter (connectivity test), air vent, heated lactated ringer for post-surgical recovery.

## Surgery

- CURRENT ANESTHESIA: Anesthetize animal with isoflurane inhalation. Turn on O2 tank (verify psi). Set O2 regulator (“4” setting).

|  | O2 flow setting | Isoflurane flow setting |
| --- | --- | --- |
| Knock down Box | “2.5” | “3” |
| In stereotaxic frame (nose cone) | “1” | “2” |

- OLD ANESTHESIA: Anesthetize animal with chloral hydrate IP injection (26G needle, 50 mg/mL concentration in ddH2O).  (110uL/10g weight dose). This equates to a dose of 5.5mg/10g = 55mg/g = 55g/kg

- When in stereotaxic frame, inject buprinex (SQ): 0.1 mL (30g mouse) and ketofen (SQ): 0.1 mL

- Monitor breathing (want constant breathing, ie no gasping). Monitor depth by tail pinch, foot pinch, and then eye blink.

- Shave head with trimmer.

- Place in stereotaxic frame. Ensure teeth/mouth in bite bar. Lightly clamp nose bar.

- Evenly affix right and left ear bars.

- Apply eye ointment.

- Adjust jaw/snout to align L & R orbital sockets.

- Tighten nose bar when straightened.

- Apply iodine to dorsal surface of head.

- Make midline cut with forceps/scissors to expose the skull. Cut and push away the periosteum (clear film/fascia).

- Expose skull surface w/ forceps. Use clean/dry cotton swab to dry surface of skull.

- Place gauze over the eyes & turn on

- Align A-P plane: For Bregma and Lambda, respectively: measure X position and Z position.

B to L:

4.21 mm per Paxinos & Franklin, Mouse Brain in Stereotactic Coordinates

Figure 1 - Rodent skull surface diagram includes the sagittal, coronal and lambdoid sutures defining the stereotaxic landmarks bregma and lambda.

- Verify alignment by measuring Bregma and Lambda coordinates. Y and Z coordinates should be the same.

- To align tilt (Y direction), position drill bit in center of B/L. Measure Z direction 2mm left and 2 mm right of center. E.g. if center = 40.0, then find Z @ 38.0 and Z @ 42.0. Proceed if Z38.0 = Z42.0.

- Input into record sheet (Excel).

- Drill holes according to stereotactic coordinates (Excel).

- Score skull with scalpel to improve adhesion to cement. This is important for long-term adhesion.

- Sequence of implants:

  - #1, #4, #6 (for Right-handers with forceps using Left)

  - #2, #3, #5

  - For single wires: position single wires with clamp arms, both right and left

- After implants are positioned, apply vetbond (medical cyanoacrylate glue) to holes. Once glue has dried (~10min), proceed to applying dental cement.

- Cement implants into place. Cure for ~15min.

- OPTIONAL:  Implant EMG electrodes. 2cm silver wire soldered to right angle pins. Glue onto left/right nuchal (neck) muscles.

- Insert pins into respective slots in block:

| #1 | #2 | #3 | #4 |
| --- | --- | --- | --- |
| #5 | #6 | #7 | #8 |

- Raise #2 silver wire and wrap twisted wires around.

- Apply dental cement. 2.5 scoops and 22 drops, use transfer pipette.

- Verify circuit with “Continuity tester” mode of multimeter.

- Cement headstage into final position. Allow 20 min of dry time.

- Inject lactated ringer soln for rehydration (0.75 mL/ 30g mouse).

# References

- Kam K, Duffy AM, Moretto J, LaFrancois JJ, Scharfman HE. Interictal spikes during sleep are an early defect in the Tg2576 mouse model of β-amyloid neuropathology. Scientific reports.

- Nicolelis MAL, editor. Methods for Neural Ensemble Recordings. 2nd edition. Boca Raton (FL): CRC Press/Taylor & Francis; 2008. Chapter 2Surgical Techniques for Chronic Implantation of Microwire Arrays in Rodents and Primates. Laura M. O. Oliveira and Dragan Dimitrov.

- Nicolelis MAL, editor. Methods for Neural Ensemble Recordings. 2nd edition. Boca Raton (FL): CRC Press/Taylor & Francis; 2008. Chapter 5 Chronic Recordings in Transgenic Mice. Kafui Dzirasa.

- Curr Protoc Mouse Biol. 2012 Mar 1;2(1):55-74. doi: 10.1002/9780470942390.mo110126. Sleep and EEG Phenotyping in Mice. Mang GM, Franken P.

- J Neurosci Methods. 2009 Mar 15;177(2):355-60. doi: 10.1016/j.jneumeth.2008.10.020. Epub 2008 Nov 1. Neck electromyography is an effective measure of fear behavior. Steenland HW, Zhuo M.

- Methods Mol Biol. 2012;821:373-91. doi: 10.1007/978-1-61779-430-8_24.

- Video-EEG monitoring methods for characterizing rodent models of tuberous sclerosis and epilepsy. Rensing NR, Guo D, Wong M.

- Curr Protoc Mouse Biol. 2012 Sep 1;2(3):273-94. doi: 10.1002/9780470942390.mo120089. In Vitro and In Vivo Recording of Local Field Potential Oscillations in Mouse Hippocampus. Forsyth LH, Witton J, Brown JT, Randall AD, Jones MW.

- Nat Protoc. 2006;1(6):3166-73. Stereotaxic gene delivery in the rodent brain. Cetin A, Komai S, Eliava M, Seeburg PH, Osten P.

- A mouse model of intracerebral hemorrhage using autologous blood infusion