Contents

Construct electrode components	1

Common REF block – 3 channels	1

Common REF block – 4 channels	2

For Single Wire Implants:	2

Cortical Screws	3

For Bipolar Twisted wires	3

Perform stereotactic EEG Implant Surgery	4

Prep	4

Surgery	6

References	11

# Construct electrode components

## Common REF block – 3 channels

- Cut 4x2 block. File down edge of 4th row to make smooth. Remove silver pins from block.

- Cut 2.0 cm silver wire.

- Solder one end to right angled tail of a pin

- Solder other end to 0.10” screw

- During surgery, insert into #1 slot.

- Repeat for #2, #3, #5.

- Insert two tails into #4 & #6 slots.

- For 4-6 common reference: twist 2 silver wires and solder contact to one right angle tail. Solder end of one silver wire to 7/8 tails and the other silver wire to a 0.10” cortical screw.

- Paint GND edge with whiteout.

## Common REF block – 4 channels

- Make 5x2 block. Remove silver pins from block.

- Cut 2.0 cm silver wire.

- Solder one end to 0.10” screw

- Solder other end to right angled gold tail

- Insert into #5 slot

- Insert two tails into #7/8 slots.

- For 5-7-8 common reference block: twist 2 silver wires and solder contact to one right angle tail. Solder end of one silver wire to 7/8 tails and the other silver wire to a 0.10” cortical screw.

## For Single Wire Implants:

- Cut 7 cm stainless steel wire

- Fold in half. Insert into block to find length of coating to remove. Fold to make crease.

- Burn off ~0.5cm from bend. This should be the depth of the micropin block.

- Sonicate to remove coating (~5min). Verify under dissection scope.

- Check Continuity

- Insert in block. Push tail-less pins into block to secure single wire.

- Use wirecutter to clip and separate wires.

- Verify circuit with “Continuity tester” mode of multimeter.

- For common reference PreAmp  insert tail-less pins into #7/#8 slot

## Cortical Screws

- Clamp right angle gold pins.

- Coat tail with flux.

- Strip ~1mm Teflon off from silver wire.

- Solder silver wire to tail of gold pin.

- Cut 1.5 cm length of silver wire.

- Strip ~2mm Teflon off from other end of silver wire.

- Curl around cortical screw head.

- Add flux to silver curl.

- Solder.

- Verify with “continuity tester”.

## For Bipolar Twisted wires

- Specifications

  - 0.0035” = 88.9 microns in diameter

  - Inspect untwisted ends to ensure void of kinks.

- Fasten wires to block

  - Heat untwisted ends to strip coating (lacquer). Use flat forceps to strip burnt coating.

  - Cut 2.5 cm from start of twist

- Make Z bend in twisted wire.

  - Using slim flat forceps, grip at slightly above 2mm. Twist and bend to make Z.

  - Want Z bend to be in one plane and near 90° bend

- Verify circuit with “Continuity tester” mode of multimeter.

# Perform stereotactic EEG Implant Surgery

## Prep

- Turn on heat sterilizer.

- Sonicate EEG implant (2 twisted wires, #1, 2, 5, 6 bone screws)

Stereotactic frame setup: mouse in frame on thermal pad. Surgical gloves, bent serrated forceps, flathead screwdriver, microscissors, and view finders. Warmed lactated ringer for post-surgical recovery (0.3 mL insulin syringe).

Isoflurane setup and connection to stereotaxic nose cone/bit bar

Implant build tools: right arm w/ drill, multimeter (connectivity test), drill bit, hemostat & plastic cup (mix grip cement); & straight forceps; cement applicator and short transfer pipette, sterile cotton swabs (2x), sterile gauze (2x), grip cement and solvent. Petroleum for eyes, Nair and hydrogen peroxide for midline scalp incision.

## Surgery

- CURRENT ANESTHESIA SETUP (changes based on OR location): Anesthetize animal with isoflurane inhalation. Pour isoflurane into vaporizer spigot and verify level (~3/4 full for 6 hrs at desired flow rate/concentration). Open O2 tank (verify psi). Set O2 regulator (“1” setting).

|  | O2 flow setting | Isoflurane flow setting |
| --- | --- | --- |
| Knock down Box | “1” | “3” |
| In stereotaxic frame (nose cone) | “1” | “2 start” |

- Turn on isoflurane settings for knockdown box, then place animal inside and seal. Start timer. Verify loss of righting and slowed breathing (1/sec).

- After ~5 min, quickly place animal in stereotaxic frame. Ensure teeth/mouth in bite bar (incisors in teeth slot and tongue flopped/exposed). Lightly clamp nose bar.

- Evenly affix right and left ear bars (~4.5mm gap on either side aka 9mm interaural distance).

- When in stereotaxic frame, inject buprinex (SQ): 0.1 mL (30g mouse) and ketofen (SQ): 0.1 mL.

- Monitor breathing (want constant breathing, ie no gasping @ ~1 breath/sec). Monitor depth by tail pinch, foot pinch, and then eye blink.

- Remove hair: Nair or Shave head with trimmer. Clean off.

- Apply eye ointment.

- Apply iodine to dorsal surface of head.

- Make midline cut with forceps/scissors to expose the skull. Cut and push away the periosteum (clear film/fascia).

- Expose skull surface w/ forceps. Use hydrogen peroxide dipped cotton swab to clean skull sutures.

- Now that skull sutures are visible, adjust jaw/snout position to align B to L plane. Inspect behind mouse, left and right (use your V1’s straightness/parallelism detection and/or the mini-level).

B to L dist:

4.21 mm per Paxinos & Franklin, Mouse Brain in Stereotactic Coordinates 2005

Rodent skull surface diagram includes the sagittal, coronal and lambdoid sutures defining the stereotaxic landmarks bregma and lambda.

- Tighten nose bar when straightened.

- Take measurements for Implant sheet: goal is to equalize YAW (snout L-R) and PITCH (snout U-D) b/w Bregma and Lambda then Roll (head rotation) between medial to lateral axis.

See airplane diagram for reference

- For Bregma and Lambda, respectively: measure X, Y and Z position as indicated on Implant sheet:

Implant sheet documents the following w/ respect to left-armed stereotaxic frame: Location (RoI), X (A to P distance), Y (M to L distance), Z (D to V distance) and drill/implant sequence.

- To align YAW and PITCH, Verify alignment by measuring Bregma and Lambda coordinates. Y and Z coordinates should be within 0.1 mm (tolerance of frame). Adjust by ear bars and/or nose cone pitch.

- To align ROLL, position drill bit in center of sagittal suture. Measure Z direction 2mm left and 2 mm right of center. E.g. if center = 40.0, then find Z @ 38.0 and Z @ 42.0. Proceed if Z38.0 = Z42.0. Adjust with ear bars.

- Record final Bregma and Lambda coordinates in Implant sheet (Excel).

- Calculate desired RoI/”Location” coordinates.

- Drill holes according to “Drill Sequence”.

- Dry skull if need be with gauze and cotton swab.

- Score skull with scalpel to improve adhesion to cement. This is important for long-term adhesion.

- Example Sequence of implants (follow desired Implant sheet):

  - #1, #4, #6 (for Right-handers with forceps using Left)

  - #2, #3, #5

  - For single wires: position single wires with clamp arms, both right and left

  - For EMG, screw in common reference electrode then create a gap above the nuchal muscles using a curved forceps. Apply vetbond to skull/nuchal muscle/EMG wire intersection.

- After implants are positioned, apply vetbond (medical cyanoacrylate glue) to holes. Once glue has dried (~10min), proceed to applying dental cement.

- Cement implants into place. Cure for ~10min.

- Insert pins into respective slots in block:

- After electrode female pins are inserted into the microconnector: verify circuit (shorting) with “Continuity tester” mode of multimeter.

- After each pin is mapped to correct electrode, cement headstage into final position. Allow 20 min of dry time: Using orange plastic cup, 2.5 scoops cement powder and ~25 drops of cement solvent. Mix well with paperclip for 10seconds. Apply on skull and electrode assembly with a small transfer pipette.

- Inject lactated ringer soln for rehydration (0.75 mL/ 30g mouse).

- Allow animal to recover in solo cage on a heated blanket (~1hr).

# References

- Kam K, Duffy AM, Moretto J, LaFrancois JJ, Scharfman HE. Interictal spikes during sleep are an early defect in the Tg2576 mouse model of β-amyloid neuropathology. Scientific reports.