| Purpose: |  |
| --- | --- |
| Date: |  |
| Coordinates | A-P; M-L; D-V. Start:. End:. |
| Weight: |  |
| Dose: |  |
| Volume Injected: |  |
| Injection type: |  |
| Notes/Bleeding: |  |

# Prepare virus

# Injection procedure

- Set up stereotaxic frame.

- Anesthetize and load/prep animal.

- Nair, inject lactated ringer, apply eye ointment

- Midline incision, H2O2 skull surface, hemostat skin

- Set up drill (on left arm), measure Bregma/Lambda, drill holes

- Lower Isoflurane to 1.5%

- Load Syringe into Kopf adapter

- Set up right arm.

- Attach syringe+adapter:

- Backfill syringe with cargo.

- Approach Right side bore hole. Zero the “Start Z” coordinate (D-V axis).

- Go to “End Z -0.02”. Hold for 10 sec. Make pocket for virus.

- Pull back to “End Z”. Inject manually.

- Wait 10 min for virus to diffuse:

- Pull back to “End +0.02”. Hold for 5 min.

- Slowly, pull out of brain.

- Reload syringe for other injection site.

- Repeat steps for Left side.

- Drop isoflurane to 1.0%.

- When done, apply small drop of cyanoacrylate glue to bore holes.

- Suture with autoclip.

# Prepare Neuros Syringe

Clean out syringe

Backfill with virus (do this when syringe+adapter is on frame)

# References

- Direct Intraventricular Delivery of Drugs to the Rodent Central Nervous System

- Optogenetic interrogation of neural circuits- technology for probing mammalian brain structures

- Stereotaxic Microinjection of Viral Vectors Expressing Cre Recombinase to Study the Role of Target Genes in Cocaine Conditioned Place Preference